XA-02 MCU’ Firmware Upgrade steps

(Note: The upgrade can only be performed on a Windows computer.)

CONNECT THE UPGRADE WIRE

1. Power on XA - 02

2. Use a USB-C cable to connect your computer's USB port to the UPDATE port of XA-02.

INSTALL BURNING TOOL ON YOUR COMPUTER

- Double-click to install the ch341ser,

- Installation...,

- Click "Install",

- Installation is successful, click "OK",

- Open the computer's "Control Panel", then open the “Device Manager”, check the port drop-down. If the device "USB-SERIAL CH340" is found, skip directly to step 6.

* If you don't find "USB-SERIAL CH340", then check “other devices”, click the drop-down to see if there is "USB Serial", and right-click to “update the driver”.

    - Then click "Browse my computer to find drivers"，

    - Click "Next step"，

    - Click "Next step"，

④ Click "USB-SERIAL CH340" and then click "Next".

⑤ Update is successful!

⑥Looking at the "port" again, you will find the device "USB-SERIAL CH340".

Start burning the firmware.

- Double - click to open the "stcai - isp - v6.94D.exe" program.

- Close the notice box.

- If the display interface is in Chinese, please click "English" to convert it to an English interface.

- Check whether the Scan Port same as USB SERIAL port.

- Click the "Check MCU " and see that the MCU Type is STC15L2K60S2.

- Click on "Open Code File" , find the firmware "XA-02_MCU_V1.3", click it to select, and then click "Open".

- Set the Input IRC frequency as“11.0592MHz”。

- Click "Download/Program" to start burning firmware.

- When it prompts "completed", it means the upgrade is successful. You can unplug the upgrade cable and enter normal use.

- You can check the firmware version in the XA-02 menu: short press the menu button, adjust the knob, and you will see the firmware version is V1.3.